# Standard Format CSV - Reference Guide

These two files are reference samples for the Standard Format used by DATO/BSI to store UT tube thickness inspection data. Both use identical structure. The only difference is tube numbering direction.

## What the format contains

Every file has four sections: a header block, a tube number row, a data block, and a legend.

The header block occupies rows 5 through 12. It stores identifying information about the inspection - the company, mill location, boiler name, inspection date, boiler section, tube count, numbering direction, and the NDE laboratory that performed the inspection. Each label sits in column F and the corresponding value sits in column H.

The tube number row sits at row 15. It lists every tube number in order across the columns starting at column F. In a left-to-right file the numbers run 1, 2, 3... ascending. In a right-to-left file they run from the highest tube number down to 1. This row is mirrored exactly at the bottom of the data block as a footer.

The data block starts at row 17. Each elevation band gets three consecutive rows - one for LEFT face readings, one for CNTR, one for RGHT. Column A holds the tech name on the first row of each band. Column C holds the elevation label on the first row of each band. Column E holds LEFT, CNTR, or RGHT. Readings start at column F and run across one column per tube, aligned to the tube number header above.

All readings are stored as 3-digit integers representing thousandths of an inch - so a wall thickness of 0.234 inches is stored as 234. Leading and trailing zeros are preserved - 0.010 inches is stored as 010, and 0.240 is stored as 240. Missing or inaccessible readings use flag characters from the legend rather than blank cells or zeroes.

The legend sits below the footer row. It defines every flag character that can appear in the data. Two blank rows separate the footer from the legend.

## The two file types

Left-to-right files have Left-to-Right in row 11 and tube numbers running 1, 2, 3... from left to right across the columns. This is the most common orientation.

Right-to-left files have Right-to-Left in row 11 and tube numbers running from the highest number down to 1 from left to right across the columns. The physical meaning is that tube 1 is on the right side of the boiler wall when facing it. The data column positions are the same - column F always holds the first tube listed, whatever number that is.

## What these sample files contain

Both samples use 20 tubes, 10 elevation bands, and fictional readings generated for illustration purposes only. The company, mill, boiler, and lab names are placeholders. The readings are random values in the typical range for boiler tube inspection. These files are not derived from any real inspection.